# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.07.00

# 02/04/2003

Release Notes and Restrictions for 05.07.00:

- No Changes from 5.02.75 except for the new version number.

Major Changes From Version 05.02.75:

General Changes

Functionality

- No changes.

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.02.75

# 01/22/2003

Release Notes and Restrictions for 05.02.75:

- Previous version was 5.02.73, skipped 5.02.74 for test

- Change in Configuration Utility

Major Changes From Version 05.02.73:

General Changes

Functionality

- Put Hot Spare support into CU, merge out in error

- Fixed a screen concatenation error for “Updating Adapter List”

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.02.73

# 12/13/2002

Release Notes and Restrictions for 05.02.73:

- Previous version was 5.02.72

- Changes were made to use 1E firmware with IM build

Major Changes From Version 05.02.72:

General Changes

Functionality

- Removed references to IOC page 3

- Added changes to work with 1E firmware

- Added ‘Initializing …..’ message to banner.  Lets user know something is happening.

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.02.72

# 10/23/2002

Release Notes and Restrictions for 05.02.72:

- Many significant changes from 5.02.00

- This build is targeted at the BASE and GENERIC binaries using many techniques and fixes from various specific builds, many of which are to be listed below.

- This binary is intended to be released as 5.07.00 after testing

- Includes these changes/fixes/improvements:

  - 17 Scatter/Gather entries

  - SAF-TE detection improved

  - Master Boot Record is wiped when mirroring is broken

  - Down arrow problem in CU is fixed

  - “Initializing all adapters” message repositioned to 0,0 location

  - Allow entry to CU is mirror is degraded

  - Zero initial BEV in placeholder BIOS (no devices to support)

  - Modify function 41h and 48h, reinterpret EDD 3.0 spec.

  - Source is initializing channel NVM (instead of FirstAdapterFound)

  - Add progress indicator for mirroring in CU

  - Refresh mirror status every 60 seconds

  - Integrated Mirroring F1 key help support removed for size

  - Fix wide behind/beyond narrow device failure

  - Fix Cluster bit in NVM, was written in error

  - Fix NVM write error, extra writes

  - Concatenate lines for warning messages

  - Revised warning messages for size and clarity

- Additionally, BASE build shows (*) and (**) for U160 and U320 capable targets.  GENERIC build shows actual negotiated speeds with independent drives.

Major Changes From Version 05.02.00:

General Changes

Functionality

- Following BIOS upgrade, the use of “Initializing channel” NVM may require CU entry and setup.

- Zeroed BEV might cause Boot Setup to change Boot Order

- Function 41h/48h changes might affect expected diagnostic results, but should not cause any error.

Defect fixes

- A Wide device behind a Narrow device caused negotiation problems, and has been fixed.  It will of course operate as a narrow device.

- The Cluster bit was being written to all channels, instead of the choosen channel.  The effect was slower recovery than necessary when an error occurred on the channel which should not have been set.

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.02.00

# 4/05/2002

Release Notes and Restrictions for 05.01.02.00:

- No changes. Merely a version number change for the GCA release.

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.01.02.00

# 3/18/2002

Release Notes and Restrictions for 05.01.02.00:

- Two changes from 5.01.00.00

- An error was fixed where a routine was called with the size in bytes when it should have been in DWords.  Added a divide by 4 to fix.

- Improve error detection during the bus scan.

Major Changes From Version 05.01.00.00:

General Changes

Functionality

- Improve error reporting during a bus scan.

Defect fixes

- Fixed a problem in a parameter that could have caused problems.

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.01.00.00

# 3/12/2002

Release Notes and Restrictions for 05.01.00.00:

- Tested with MPT SCSI firmware 00.00.10.00 and later.

- BIOS runs at Fast-40 speeds only. Logic will detect Fast-80 (Ultra160) and Fast-160 (Ultra320) capability.  The GENERIC build will negotiate for U320 with Domain Validation.

- Displayed negotiation speed for the last device displayed may erronously show half the actual speed.

- May not work with legacy boot loader on EFI systems.

Major Changes From Version 05.00.00.00:

General Changes

Functionality

- If the build is GENERIC, code will recognize SAF-TE/SES devices, and avoid negotiations with them.  Fixes problem with enclosures that caused those devices to be dropped from the discovery list, and prevented proper negotiations with other devices.

- If the build is GENERIC, temporary negotiation problems will not be written to non-volatile memory, and each successive boot will attempt negotiations at default settings, rather than at degraded safer settings which required going into the CU to set back.

- If the build is GENERIC , CHS display is replaced with CAPACITY in Mbytes.

- If the build is GENERIC, BBS expansion header information has an additional character to show the bus in addition to the device/function.  Display begins with #XXX instead of #XX with the first X indicating the bus number.

- If the build is GENERIC, the banner no longer shows an explanation for the * and ** flags.

Defect fixes

- None

# Release Notes

# MPT BIOS and Configuration Utility

# Version 05.00.00.00

# 3/11/2002

Release Notes and Restrictions for 05.00.00.00:

- Tested with MPT SCSI firmware 00.00.10.00 and later.

- BIOS runs at Fast-40 speeds only. Logic will detect Fast-80 (Ultra160) and Fast-160 (Ultra320) capability

- Displayed negotiation speed for the last device displayed my erronously show half the actual speed.

- May not work with legacy boot loader on EFI systems.

Major Changes From Version 04.18.09.04:

General Changes

Functionality

- 64K Initialization Memory was being wasted by allocation code. Changed allocation routine to use actual size instead of carryover artificial limit.

- Code will recognize SAF-TE/SES devices, and avoid negotiations with them.  Fixes problem with enclosures that caused those devices to be dropped from the discovery list, and prevented proper negotiations with other devices.

- Banner will display the full FW version in place of a truncated version.

Defect fixes

- Fixed initialization memory allocation routine.

- Fixed line counter in banner display  to properly output device data when devices require more than one screen for display.

- Fixed problem with hangs and negotiations when devices are in an enclosure.

# Release Notes

# MPT BIOS and Configuration Utility

# Version 04.18.09.04

# 3/01/2002

Release Notes and Restrictions for 04.18.09.04:

- Tested with MPT SCSI firmware 00.00.09.00 and later.

- BIOS runs at Fast-40 speeds only. Logic will detect Fast-80 (Ultra160) and Fast-160 (Ultra320) capability.

- Approximately 15 devices print on POST screen.

- May not work with legacy boot loader on EFI systems.

Major Changes From Version 04.18.09.03:

General Changes

Functionality

- Initialize Free Memory allocation from 32KB to 64KB if PMM doesn’t exits.

Defect fixes

- Pause during Boot Alert.

##### VERSION 04.18.09.03

Major Changes From Version 04.18.09.02:

General Changes

Functionality

- Add code so IM BIOS will disable IM support if future IR firmware doesn’t support IM.

- Disable “Force Synchronization” button if IM Secondary disk no online.

###### Defect fixes

- Fix termination control logic.

##### VERSION 04.18.09.02

Major Changes From Version 04.18.09.01:

General Changes

Functionality

- None.

Defect fixes

- Firmware download boot changes to properly detect MAD strapping and correct redundant subroutine entries.

- Add changes to adapter reset routine specified by Firmware Team.

##### VERSION 04.18.09.01

Major Changes From Version 04.18.09.00

General Changes

Functionality

- None.

Defect fixes

- Corrected memory de-allocation error.

##### VERSION 04.18.09.00

Major Update Release

General Changes

Functionality

- Provide special builds and hooks for customer specific requirements.

Defect fixes

- None.

### NOTES, ISSUES and DETAILS

- SES devices (identified with 0x0D) aren’t excluded as are SCSI Processor devices. Sync negotiations will hang.

- CD-ROM booting fails on systems that don’t keep the floppy on the INT40h chain when a hard disk drive is not present.

- HBA’s report full speed and not actual speed (e.g., when a single-ended drive is on the bus).

- Can’t modify data width in BIOS (change not acted on).

- If the first drive on the fifth controller is the first drive scanned, it won’t boot. If it’s the third drive scanned, after two on the first controller, it will boot.